Swim England Environmental Assessment Tool

Third Edition (May 2021)

Is your facility accessible and inclusive?

For people with long term health conditions and impairments.

Introduction

Environmental analysis and change tool for facilitating people with long term health conditions and impairments

Swim England Health Model

It is important to consider both the physical and social environment of any public or community building on order to identify areas of improvement. There are a range of things leisure facility providers can do to help people to remain engaged in physical activity in a meaningful way. This can be done by ensuring the physical environment is easy to navigate and safe for people living with long term health conditions or impairments.

How to use this tool in leisure centre environments

Community leisure facilities represent very unique and challenging environments for those with long term health conditions and impairments. They can be busy and noisy, with the added challenge of gym and pool smells as well as varying light levels and both wet and dry flooring.

This checklist is intended to be used by individuals and groups interested in making the swimming experience more accessible for the local and extended community. In assessing a leisure facility’s strengths and deficiencies, people living with long term health conditions and impairments can describe how the checklist of features matches their own experience of the centre’s positive characteristics and barriers. Wherever possible they should play a role in suggesting changes, through constructive conversations, and implementing and monitoring improvements.

This environmental analysis and change tool has been specially designed to support you in creating safe and friendly aquatic environments. It has been designed to be practical and easy to use and focuses on those aspects of the physical environment known to impact on those with long term health conditions and impairments. It assesses both the physical environment and the way the environment encourages people to behave and interact. It may also be helpful to take photographs as a part of the process as these can provide a useful ongoing reference tool to monitor progress.

How you might use the results

Before  carrying  out  the  assessment  please  ensure  that  you have  the relevant  management  backing to build support and commitment to the results. You could benchmark your scores against other leisure Centre’s in your region to see how you compare to highlight particular areas for improvement.

If you have low scores in a particular area, think what action you can take to make improvements that are within your area of control and those that will need the right support of others. If your score are low overall, you might want to use this to help inform discussion on the need for environmental improvements with senior management.

Essential elements

The elements of each section deemed as ‘essential’ have been highlighted in ‘BOLD’ Whilst some elements might be able to be worked around, some would arguably pose a barrier to access or result in a very negative experience for anyone and therefore should be prioritised in any action plan around improvement.

Section A - Outside entrance and arrival

A clearly signposted, and visually obvious, entrance is key to simplifying the beginning of the leisure centre experience, especially if the building is located in a busy urban area amongst other community facilities. It’s important that from the point of arrival, directions and design cues clearly lead the way to the leisure centre entrance.

Please score each answer 1, 3 or 5 (1=not met, 3=partially met, 5=totally met)

| - Is the entrance to the Leisure Centre clearly signposted from the nearest bus stop and car park? |  |  |
| --- | --- | --- |
| - Is accessibility clear in marketing material and website information to reduce any anxiety before coming to your centre? |  |  |
| - Are there clear signposts in the reception area about where to go? |  |  |
| - Is there a ‘who is who’ board for staff in reception? |  |  |
| - Are there clearly marked disabled bays? |  |  |
| - Does the approach to the building look and feel welcoming and inviting? |  |  |
| - Is the outside of the centre clean, tidy and cared for? |  |  |
| - Is the entrance door to the building clearly signed and visible? |  |  |
| - Are glass doors clearly marked open or closed if clear? |  |  |
| - Is there an unobstructed ramp with a clearly distinguishable handrail? |  |  |
| - Are there drop kerbs? |  |  |
| - Are there any unmarked steps into the building or other trip hazards? |  |  |
| - Is the main door easy to open without strenuous pushing or pulling? |  |  |
| - For disabled access doors, is the push button immediately obvious? |  |  |
| - Is the doorway free of a dark coloured or black floor mat that could be mistaken for as a hole? |  |  |
| Total score section A |  |  |

Section B – Lobby and reception

A clearly defined lobby and manned reception desk create a re-assuring environment for anyone with a disability, long term health condition and carers, to begin their leisure centre experience. Staff should be available to recognise and support anyone who may need additional assistance. Entrance and exit gates and barriers should be clearly distinguishable to help direct movement in the right direction.

Please score each answer 1, 3 or 5 (1=not met, 3=partially met, 5=totally met)

| - Is there an obvious main reception/enquiry desk manned with someone to welcome you or tell you where to go? |  |  |
| --- | --- | --- |
| - Is there a low level counter? |  |  |
| - Are any security gates easily accessible and simple to pass through? |  |  |
| - Are there hearing loop signs? |  |  |
| - Is there a large face clock easily visible in all areas? |  |  |
| - Is there sufficient lighting? |  |  |
| - Is there an obvious seating area with a choice of seating including chairs with arms? |  |  |
| - Is there a quiet room or a room that people can use if distressed? (could be a multi-purpose faith/reflection room) |  |  |
| - Are there well-lit signs, at eye-level, directing you to different areas of the leisure centre? |  |  |
| - Are spaces free of clutter and notices and posters kept to a minimum to avoid distraction and confusion? |  |  |
| - Are changes in floor finish flush rather than stepped? |  |  |
| - Are floor finishes of a consistent colour avoiding black stripes and dark colours creating visual barriers on the floor? |  |  |
| - Is the way out and the fire exit clearly marked and easy to see? |  |  |
| - Do you have a café or other social area? |  |  |
| - Are “staff only “doors disguised e.g. by painting them in the same colours as the walls to help them blend in? |  |  |
| - Is change required for the lockers? If so are people aware of this beforehand and is there somewhere to get change if they don’t have any? |  |  |
| Total score section B |  |  |

Section C – Wayfinding to changing rooms

Leisure centres are complex environments with numerous routes inside the building leading to different spaces for a wide range of activities. To ensure that those that have arrived for swimming know where to go once they have passed through the lobby and reception, wayfinding should be carefully and clearly considered to ensure that participants know where to move to next.

Please score each answer 1, 3 or 5 (1=not met, 3=partially met, 5=totally met)

| - Are corridors at least 2 metres wide to enable those less ambulant, and wheel-chair users, to safely pass oncoming people? |  |  |
| --- | --- | --- |
| - Do handrails contrast with the walls and can they be grasped properly? |  |  |
| - Whenever you have to make a choice about which direction to take – are there signs to help you decide which way to go? |  |  |
| - Are the signs to the changing area clearly visible? |  |  |
| - Is signage clear, in bold face, with good contrast between text and background, using words and pictures or symbols? |  |  |
| - Are signs at eye-level (approx. 1400-1700 cm high) and well lit? |  |  |
| - Are there landmarks or visual cues in the leisure centre (such as artworks or plants) that help with wayfinding and return wayfinding? |  |  |
| - Is the flooring matt, rather than shiny, in a colour that contrasts with the walls? |  |  |
| - Is the flooring a consistent colour i.e. does not have speckles, pebble effects, stripes or patterns? |  |  |
| - Could the lighting or natural light from the windows make the floor appear to be wet or slippery or cause lighting glare/flare? |  |  |
| Total score section C |  |  |

Section D – Changing areas

On arrival in the changing area, swimmers will want to go any one, or more, of the following spaces – a locker, a changing room or the toilets or wash areas. Signage and lighting need

to support wayfinding and reduce the possibility of confusion and agitation. Clear signs to the changing facilities and to the pool should be visible using distinct images and high visibility colours. A separate allocation of lockers and changing areas for dementia users will help reduce the challenges that arise from the experience of blocks of lockers and changing cubicles that all look the same.

Please score each answer 1, 3 or 5 (1=not met, 3=partially met, 5=totally met)

| - Are there clear, well-lit signs directing you to the toilets, showers and the pool? |  |  |
| --- | --- | --- |
| - Are areas clear of rubbish, not used as storage? |  |  |
| - Are doors not to heavy? |  |  |
| - Is the lighting even without creating pools of light and dark areas, stripes or dark shadows which can be confusion or disorientating? |  |  |
| - Are there changing cubicles where an opposite sex carer or partner can help out the person with their clothes? If not, are staff briefed and available in how to meet this need with dignity and privacy? |  |  |
| - Are the banks of lockers clearly distinguishable from each other, either by colour or signage or – is there a separate bank of high vis distinguishable lockers? |  |  |
| - Are the locks on the lockers simple to use and can they be opened and closed with ease? |  |  |
| - Are locker and key identifications/wrist bands easy to read and easily matched by clearly visible numbers, images or colours on both? |  |  |
| - Is the floor regularly mopped and cleared of puddles of water left from other pool users? (Check any rota) |  |  |
| - Is the exit to reception clearly visible from inside the changing area? |  |  |
| - Are the lockers close to the changing rooms? |  |  |
| - Are the changing rooms large enough for someone who might have mobility challenges to move around without feeling restricted? |  |  |
| Total score section D |  |  |

Section E – Toilets and showers

Using recognisable fixtures and fittings can help to create a familiar environment that is welcoming whilst presenting a sense of comfort and re-assurance. Signage to the different areas should be very visible, colours can be used to distinguish floors and walls and careful consideration should be given for sanitary ware for the visually impaired. Cleanliness is paramount, especially in these high traffic areas where swimmers will regularly use facilities during or immediately after a swim creating a potential falls hazard through wet, slippery floors.

Please score each answer 1, 3 or 5 (1=not met, 3=partially met, 5=totally met)

| - Can the signs to the toilets and showers clearly be seen from all areas? Are alarm cords in place and not tied? Are shower adjustable? Do you have benches in place? Do you have space for wheelchairs to turnaround with doors opened and closed? |  |  |
| --- | --- | --- |
| - Are all the toilet doors painted in a single distinctive colour and do they all have the same clear signage? |  |  |
| - Are floor surfaces non-slip and non-reflective to reduce glare? |  |  |
| - Are the toilet seats, flush handles and grab rails in a colour that contrasts with the toilet, walls and floor? |  |  |
| - Are the toilet flush, basin taps and shower taps of a familiar design, such as cross headed taps, rather than sensor controls? |  |  |
| - Are basin and shower taps clearly marked hot and cold, and water flow clearly marked on and off? |  |  |
| - Are the toiler roll holders of a familiar design and easily reached from the toilet? |  |  |
| - Are hand dryers clearly signed with words and symbols? |  |  |
| - Is there a peg outside the shower cubicle to hang a towel and toiletries bag? |  |  |
| - Is there a toilet cubicle big enough to allow space for a wheelchair and/or carer with the door closed? |  |  |
| - Is there a clothes hook inside the toilet door for clothes or towels? Is it at the correct level? |  |  |
| - Is lighting sufficient within the toilets for someone with sight restrictions? |  |  |
| - Is the way back to the changing cubicles and lockers clearly visible from the toilet and shower area? |  |  |
| Total score section E |  |  |

Section F – The swimming pool

The swimming pool can be a daunting space to enter for any swimmer, especially if it is crowded and noisy. Staff should be alert and aware of anyone that requires support and should be ready to coach anyone on ways to safely enter the pool, either via steps, ladders or ramps, especially if less able swimmers don’t want to use the hoist.

Please score each answer 1, 3 or 5 (1=not met, 3=partially met, 5=totally met)

| - Is signage and access to the pool clearly visible from the changing area? |  |  |
| --- | --- | --- |
| - Do you have a hoist, ramp or chairs to the pool? |  |  |
| - Is there someone at the poolside who is available to assist swimmers with dementia in a sensitive and appropriate manner, or an arranged process in place for certain times of the day? |  |  |
| - Is there somewhere clearly visible for swimmers to hang towels or robes, and to leave flip flops or pool shoes? |  |  |
| - Are swimmers allowed to wear t-shirts/appropriate cover ups in the pool? Is this communicated in the material from website/imagery to poolside and included on staff training? (body image issues/self-harm etc) |  |  |
| - Are the entrance steps to the pool clearly visible and can you see where they enter the water? |  |  |
| - Are any sloping ramps in the pool clearly distinguishable from any steps? |  |  |
| - Do you have ramps or any alternative way to those outlined in Q2 above that would enable someone with mobility/pain issues to ease their way into the pool? (e.g. graduated steps / beach entry access). |  |  |
| - Is there something for swimmers to hold onto around the perimeter of the pool if they wish to rest during a swim session? |  |  |
| - Are signs leading back to the changing area, showers and toilets clearly visible? |  |  |
| - Are all potentially hazardous items e.g. cleaning materials, securely locked away and doors to technical and plant areas locked? |  |  |
| - Is there a mechanism/plan for helping people in and out of the pool if they are in physical or mental distress and can’t pull themselves up the steps out of the water? |  |  |
| Total score section F |  |  |

Summary

Please transfer your total scores from each section to the table below, and add them for an overall score. Please write the name(s) of the reviewer in the box below as well as the date the tool was completed.

| A Outside entrance and arrival |  |  |
| --- | --- | --- |
| B Lobby and reception |  |  |
| C Wayfinding to changing rooms |  |  |
| D Changing areas |  |  |
| E Toilets and showers |  |  |
| F The swimming pool |  |  |
| Total score |  |  |

This is a tool to support you in how you look at your leisure centre environment. The analysis tool can be used for an initial assessment as to how accessible your leisure centre environment is, and with the scoring mechanism, can be used as a periodic review tool to assess and measure improvements.

It would be really helpful if a number of environmental assessments are undertaken by different members of staff as well as people with a disability and/or a long term health condition.

There is no perfect score or perfect outcome. The idea is to use the tool to sensitise those responsible for pools and leisure centres to the hazards and opportunities that these very specialised environments present.

Declaration:

The assessment scoring has been carried out as accurately as possible and with the aim to improve the overall experience for people with long term health conditions and impairments. We are committed to making the key improvements resulting from this assessment.

| Name: |  | Title: |  | Organisation: |  |
| --- | --- | --- | --- | --- | --- |
| Date: |  | Signature: |  |  |  |

Counter-signed by Line-Manager of the above

| Name: |  | Title: |  | Organisation: |  |
| --- | --- | --- | --- | --- | --- |
| Date: |  | Signature: |  |  |  |

Top ten tips

Please score each answer 1, 3 or 5 (1=not met, 3=partially met, 5=totally met)

| - The entrance to the building should be clearly signed and visible. |  |  |
| --- | --- | --- |
| - The reception area and receptionist should be clearly visible and easily accessible. |  |  |
| - The signs to the changing facility should be clearly visible. |  |  |
| - The flooring should be matt, in a colour that contrasts with the walls, and of a consistent colour i.e. does not have speckles, pebble effects, stripes or patterns. |  |  |
| - Banks of lockers should be clearly distinguishable from each other, either by colour or signage. |  |  |
| - Toilet seats, flush handles and grab rails should be in a colour that contrasts with the toilet, walls and floor. |  |  |
| - A selection of changing and toilet cubicles should be spacious enough for an opposite sex carer or partner to help out the person with their clothes. |  |  |
| - Signage and access to the pool should be clearly visible from the changing area and the changing area should be well signposted from the pool. |  |  |
| - Steps and ramps into the pool should be sign-posted well and clearly visible as they enter and leave the water. |  |  |
| - All exits and ways out from the pool, changing areas, and reception should be clearly signposted. |  |  |